AI-Based Morse Code Translator and Decoder.

### Team Members

DIVYANSH VERMA

FARHAN ALAM

VIRAL GUPTA

Abstract:

This project demonstrates a hybrid hardware–AI system that enables real-time communication through Morse code. A user inputs a text message, which is converted into Morse code and transmitted as LED blinks using an Arduino. A laptop webcam captures these LED blinks, and an AI-based vision model decodes the Morse code back into readable text.
The project showcases the integration of embedded systems, computer vision, and AI-based decoding, illustrating how traditional communication systems can be enhanced using modern intelligent technology.

Objectives:

  To design a system that converts text into Morse code signals using Arduino.

  To transmit Morse code visually using LED blinking patterns.

  To use a webcam and AI/Computer Vision model to decode the Morse signals back into text.

  To demonstrate the fusion of hardware and AI for secure, light-based communication.

## Block Diagram Explanation

- User Input Text:
The user types a message through the serial monitor or Python interface.

- Morse Code Encoder:
The text message is converted into Morse code using a lookup table.

- Arduino Controller:
Arduino receives the Morse code sequence and blinks an LED accordingly — short blinks for dots, long for dashes, and pauses for spaces.

- LED Transmitter:
The LED acts as a signal transmitter that sends visible light signals representing Morse code.

- Laptop Webcam (Receiver):
The webcam continuously captures the blinking LED and passes the video feed to the computer vision algorithm.

- AI/Computer Vision Decoder:
OpenCV (or an AI model) analyzes frame brightness to detect ON/OFF durations and reconstructs the Morse code sequence.

- Morse-to-Text Conversion:
The detected Morse code pattern is decoded into alphanumeric text using a dictionary.

- Output Display:
The final decoded message is displayed on the laptop screen — matching the original input.

### Components Used

Hardware:

- Arduino Uno / Nano

- LED (with resistor)

- Jumper wires

- Breadboard

- Laptop webcam

Software:

- Arduino IDE

- Python (with OpenCV, NumPy, and custom AI logic)

- Serial communication via USB

- Morse code mapping algorithms

### System Architecture

Step 1: User enters a message in the serial monitor or Python interface.
Step 2: The message is converted into Morse code (dots and dashes).
Step 3: Arduino controls the LED to blink short (dot) and long (dash) signals.
Step 4: Laptop webcam captures LED blinking in real-time.
Step 5: The Python script processes the video feed using OpenCV to detect light ON/OFF durations.
Step 6: AI/logic model interprets the timing pattern to identify Morse code.
Step 7: Decoded Morse is converted back into text and displayed on screen.

### Working Principle:

- Encoding:
The Arduino code takes text input and converts each character into Morse code using a predefined dictionary. LED blinks short (for dot) and long (for dash) signals with pauses in between letters and words.

- Transmission:
The LED acts as a transmitter that encodes data into visible light.

- Decoding:
The laptop webcam continuously monitors the LED. Using frame intensity analysis, the program detects when the LED is ON or OFF and measures the duration of each state.
These durations are mapped to dots, dashes, and spaces, reconstructing the Morse message.

- AI Integration:
The decoding system can use a thresholding algorithm or a trained model that learns timing variations (due to lighting or frame rate) to accurately predict Morse symbols.

### Applications:

- Secure light-based communication.

- Communication in low-signal or silent environments.

- Learning and demonstration of AI + hardware integration.

- Assistive technology for visually impaired communication.

- Educational tool for embedded-AI collaboration.

### Future Enhancements

- Integrate wireless communication between devices.

- Use deep learning (e.g., CNN or RNN) to automatically detect Morse code from light patterns.

- Add sound-based decoding using microphones.

- Display decoded messages on an OLED screen.

- Optimize timing calibration using adaptive learning.

### Conclusion:

This project successfully integrates hardware (Arduino) and AI (Computer Vision) to enable a real-time Morse code communication system. It demonstrates how classic communication methods can be modernized using artificial intelligence, showcasing an innovative fusion of electronics, signal processing, and machine learning.